| Assignment | : | Systems Programming and Computer Control |
| --- | --- | --- |
| Subject Code | : | CT047-3-2-SPCC |
| Date Assigned | : | Week 3 |
| Date Due | : | Week 10 |

A company has received a contract to design a controller car wash. The specifications and rules are listed below. Your job is to design a controller using LabVIEW that satisfies all specifications.

Instructions:

- Document your assumptions either in your Document or on the LabVIEW block diagram.

- The assignment requires you to develop a LabVIEW application based on a set of specifications.

- A computer with a standard installation of LabVIEW is the only reference allowed for the assignment. Externally developed code and third party tools are not allowed in the assignment.

- You are to submit your finished application online via MS Teams.

- Total time allocated for the assignment is 7weeks

Grading:

The application development assignment consists of a total of 50 points (100%) which are allocated as follows:

| Criteria | Points | Percentage % |
| --- | --- | --- |
| Programming style | 15 | 30 |
| Functionality | 15 | 30 |
| User Interface Design | 5 | 10 |
| Conformity to design standards | 5 | 10 |
| Documentation | 10 | 20 |

A presentation will follow to allow students to showcase the full functionality of their application. The goal of this presentation is to ascertain that the application submitted is the intellectual product of the student.

IMPORTANT:

- When you have completed the Assignment, submit the assignment document as a MS Word Document, and the S/W (along with any other resource) as a Zip file

- Submit the files on MS Teams.

Application Development

Section I: General Requirements

The application should:

- Function as specified in Section II of this document.

- Conform to LabVIEW coding style and documentation standards (found in

LabVIEW documentation – LabVIEW Development Guidelines).

- Be created using VIs and functions available in LabVIEW (templates, examples, or code developed outside the core functions of LabVIEW are not acceptable).

- Be hierarchical in nature. All major functions should be performed in sub VIs.

- Use a state machine that either uses a type defined enumerated control, queue, or

Event structure for state management.

- Be easily scalable to add more states / features without having to manually update

the hierarchy.

- Minimize the use of excessive structures, variables (local / global) and Property

Nodes.

- Respond to front panel controls (within 100 ms) and not utilize 100% of CPU

time.

- Close all opened references and handles where used.

- Be well documented and include the following:

- Labels on appropriate wires within the main VI and sub VIs.

- Descriptions for each algorithm.

- Documentation in VI Properties » Documentation for both main VI and sub VIs.

- Tip strip (tool tip) and Description for front panel controls and indicators.

- Labels for constants.

Section II: Application Requirements Security System

Instructions:

Using a front panel similar to the graphic provided, create a LabVIEW application that implements a car wash controller.

General Operation:

The car wash controller simulates the control system of an automated car wash. The user interacts with controls and indicators on the front panel to select the car wash options and simulate the travel of the vehicle in the car wash.

The controller should perform the following general operations:

- Indicate on the Wash Entry LED if the car wash is vacant or a wash is in progress.

- Allow the user to select Wash Options

- Start the washing process when the user clicks Start

- Indicate the current wash step on the Car Wash Indicators LEDs and display the Elapsed Time on the indicator

- Turn ON the Vehicle Out of Position LED and pause the elapsing time if the vehicle moves from the designated position during a cycle

Sequence of Operation:

Start (Application Run): When the application starts, the front panel controls and indicators should be in the following states:

- Entry Console: All controls should be enabled.

- Car Wash Display: The Wash Entry LED should be green and display Wash Vacant. All Car Wash Indicators should be turned OFF and the Elapsed Time indicator should display 0.00.

- Car Position Simulation: The Car Position Slider should be at the Entry position.

Select Wash Options: Click the Wash Options buttons to select the washing steps. The buttons should remain clicked until the completion of the wash cycles.

Start: Click the Start button to begin the car wash process. This should configure the controller to execute the steps selected by the Wash Options buttons.

Note The High Pressure Wash step is the default wash. If the user did not select High Pressure Wash, or if the user did not select any options, High Pressure Wash must be added to the options programmatically after the user clicks Start.

The controller should disable the Wash Options buttons.
The controller should signal the start of the wash by changing the colour of the Wash Entry LED to red and displaying Wash In Progress.
The controller should check if the vehicle is at the appropriate station for the first step in the selected wash type by monitoring the Car Position Slider.

Note Refer to the Car Wash Step Timing and Station Positions table at end of the specification for a list of stations that correspond to the car wash steps.

If the vehicle is not at the appropriate station, the controller should turn ON the Vehicle Out of Position LED and not begin timing.

If the vehicle is at the appropriate station, the appropriate wash step LED should turn ON and the Elapsed Time indicator should count up from zero to indicate elapsed time for that step.

Upon completion of a step, the Elapsed Time indicator should reset to zero.
Each wash step should not last for more than the time allocated in the Car Wash Step Timing and Station Positions table at the end of the specification.

Upon completion of a step, if the vehicle is in the appropriate station, the controller should continue to the next wash step by resetting and restarting the elapsed timer.

Upon completion of a step, if the vehicle is not in the appropriate station, the controller should turn ON the Vehicle Out of Position LED and not begin timing.

If the vehicle moves away from the wash station in the middle of a wash step, the elapsed time should pause, the Vehicle Out of Position LED should turn ON and the appropriate wash step LED should turn OFF. When the vehicle position is restored to the correct position by moving the Car Position Slider, the Vehicle Out of Position LED should turn OFF, the appropriate wash step LED should turn ON, and timing should continue from the paused time until completion of the step.

When all the wash steps are complete, the Vehicle Out of Position LED should turn ON, indicating to the user to exit the car wash. When the user moves the Car Position Slider to the Exit position, all Car Wash Indicators should turn OFF and the Car Position Slider should reset to the Entry position. The Wash Entry indicator should turn green and display Wash Vacant, and the controls in the Wash Entry Console should be enabled to allow the user to select and start another car wash process.

Stop Program: Click Stop Program to abort the car wash process at any time and stop the application. When the application stops, the front panel controls and indicators should be in the following states:
Entry Console: All controls should be enabled.

Car Wash Display: The Wash Entry LED should be green and display Wash Vacant, all Car Wash Indicators should be turned OFF, and the Elapsed Time indicator should reset to zero.
Car Position Simulation: The Car Position Slider should reset to the Entry position.

Description of Controls/Indicators:

| Control Name | Control Description - Function |
| --- | --- |
| Wash Options | Cluster of Buttons - Allows the user to customize the car wash |
| Start | Button - Allows user to start car wash process |
| Car Position Slider | Horizontal Pointer Slide - Allows user to simulate travel of car during the car wash process. Note Use Text Labels for Scale Markers Labels; Values. Entry; 0. Station 1; 1. Station 2; 2. Station 3; 3. Exit; 4. Note Refer to the front panel figure provided for car wash steps corresponding to the stations |
| Stop Program | Button - Stops the program |
| Wash Entry | Square LED - Indicates if the car wash is operational OFF Colour: Green OFF Text: Wash Vacant ON Colour: Red ON Text: Wash In Progress |
| Car Wash Indicators | Cluster of square LEDs - Indicates the current step in the wash cycle Cluster Order; Square LED Label. 0; Under Body Wash. 1; Bug Remover. 2; Pre-Soak. 3; High Pressure Wash. 4; Low Pressure Wax. 5; Spot Free Rinse. 6; Tire Shine Foam. 7; Air Dry. 8; Vehicle Out of Position. |
| Elapsed Time | Numeric. Representation: DBL, Digits of Precision: 2 |

Car Wash Step Timing and Station Positions:

| Wash Steps | Step Time | Station |
| --- | --- | --- |
| Under Body Wash Bug Remover Pre-Soak High Pressure Wash Low Pressure Wax Spot Free Rinse Tire Shine Foam Air Dry | 5 seconds 5 seconds 5 seconds 5 seconds 5 seconds 5 seconds 5 seconds 5 seconds | Station 1 Station 1 Station 2 Station 2 Station 2 Station 2 Station 3 Station 3 |

File Logging

The Car Wash System should produce an ASCII disk file in a spreadsheet-readable format on a remote Server. The data, when imported into a spreadsheet, should be in the following format: (where XXXX represents logged data)

| Date | Time | Type of Wash |
| --- | --- | --- |
| XXXXXXX | XXXXXXX | XXXXXXX |
| XXXXXXX | XXXXXXX | XXXXXXX |

The log file should be stored in a location relative to the location of the Car Wash System and should be updated (appended) only when a job is completed.

### Marking Criteria

The assignment will contribute 40% towards the entire grade of the Module, as mentioned in the Module Descriptor.

Distinction

Demonstrated comprehensive research with detailed evidence. High level of analysis performed, exceptional and thorough knowledge and understanding displayed with regard to application. This includes analysis and evaluation of facts followed by results of evaluation. Documentation presented in a professional manner, following proper sequencing and flow. Displayed evidence of critical appraisal. High level of listing citation and references.

Credit

Adequate research conducted with fair detail of evidence presented. Moderate level of understanding, analysis and knowledge displayed. Some level of relevance included in terms of application. Moderate level of analysis and evaluation of facts followed by results comparison. Good level of documentation presented.  Some level of reflection was evident in the documentation. Moderate level of listing citation and references.

Pass

Low level research conducted. Some evidence of research displayed. Basic level of understanding and knowledge analysis displayed. Satisfactory level of documentation. No evaluation and analysis of facts, no results comparison performed Satisfactory or low level of reflection displayed. Low level of listing citation and references.